CHENGDU HONELINKS INNOVATION TECHNOLOGY CO.,LTD.
Tel:0086-028- 86661062    Emai:sherry@honelinks.com  Mobile: 86-15196637014

	10G Uplink 28SFP+8GE L3 Managed Switch


	[image: image1.jpg]24*1G SFP(8COMBO)+4*10G SFP Converge Switch

1000M

[ B |
LR

[ ]
3
[ ]

[ ]
9
]

1 13
L I |

15
[ ]

17
[ ]

" ==
19
[ ]

[ ]
1
]

Link/.
[ I |
12 14

[ ]
16

[ ]
18

1000M

[10G;
L B |

Console







	Overview HONE-S6800-368SX-4

	Ports
	10G Uplink 28SFP+8GE 

	Fixed Ports
	16*1000M SFP

8*10/100/1000M RJ45(COMBO)

8*100/1000M SFP(COMBO)

4*100/1000/2500/10000M SFP

	Manage ports
	1 pc console port

	Reset Key
	1 pc

	CPU
	MIPS-34Kc 800GHz

	DDR
	4G DDR3

	FLASH
	512MBytes

	RAM
	512MBytes

	Band width
	512Gbps  

	Packet forwarding
	95.23Mpps

	MAC Address
	16K

	Buffer
	12 Mbit SRAM Packet Buffer

	Jumbo frames
	Jumbo frame up to 12KB

	VLAN Address
	4K

	Link Aggregation

	Supports 128 trunk groups with up to 8 members per group

	L3 Routing
	IPV4 supports 6K; IPV6 supports 2K；

	QoS Queue
	8

	Transmission distance
	PWR:Power LED

SYS:System LED

1-24:10/100、1000M Network connection indicator

25-28:10G SFP connection indicator

	LED indicator 
	PWR:power indicator light

SYS: system indicator light

9-10:SFP Optical port connection indicator light

PoE Porst: Orange PoE indicator light, green network connection indicator light

	Power supply
	Built-in redundant dual power supply max 45W 

AC:100-240Vac 50-60Hz 0.6A

	Working temperature/humidity
	-10～+55°C；5%～90% non-condensing

	Storage temperature/humidity
	-40～+75°C；5%～95% non-condensing

	Product/package size (L*W*H)
	440mm*255mm*45mm

515mm*375mm*95mm

	Net weight/gross weight (kg)
	3.6kg/4.5kg

	Lightning/surge protection level
	6KV 8/20us；

IP30

	Installation method:
	Rack-mount (optional machine hanger spare parts)

	Certification:
	CE-EMC EN55032；CE-LVD EN62368；

FCC Part 15 Class B；RoHS；

	Warranty period:
	Whole device for 1 year(Accessories not included)


	Features
	L3 Managed Switch

	Supported switch chips
	RTL838X

	Port

	Number of Fiber Gigabit ports
	Maximum 24 ports

	Number of Copper Gigabit ports
	Maximum 8 ports

	Number of Fiber 10 Gigabit ports
	Maximum 4 ports

	Port auto-negotiation
	Support port auto-negotiation Support forced port mode and rate

	Port Speed
	Support autonegotiate, full-1000, full-100, half-100, full-10, half-10

	Port flow control
	Supports full-duplex IEEE 802.3x flow control and half-duplex back pressure flow control

	Broadcast storm control
	Support rate limiting for broadcast, multicast and DLF packets

	Port storm suppression
	Supports detection of broadcast packets, multicast packets and unicast packets of ports respectively, and closes ports when the threshold is exceeded.

	Port Mirror
	Support

	Port Speed Limit
	Support port input and output port speed limit

	Link Aggregation
	Support

Supports up to 32 aggregation groups, each group supports up to 8 ports

Supports routing strategies of source MAC, destination MAC, source and destination MAC, source IP, destination IP, source and destination IP

	Port Isolate
	Support

	Jumbo Frame
	Support up to 16KB packet

	Redundant Port
	Support

	MAC

	MAC Table Capacity
	16K

	MAC Table Management
	Support

	Forwarding mode
	Support IVL forwarding mode

	Static MAC Address
	Support

	MAC Address Binding
	Support

	MAC Address Filtering
	Support

	MAC Learning Control
	Control the MAC learning based on port

	VLAN

	Number of VLANs
	4K

	802.1q-based VLAN
	Support

	MAC-based VLAN
	Support

	IP-based VLAN
	Support

	Protocol-based VLAN
	Support

	PVLAN
	Support

	Voice VLAN
	Support

	VLAN Mapping
	Support 1:1 mapping

	QinQ
	Support basic QinQ

Support flexible QinQ

	Reliability

	Spanning Tree Protocol
	Support STP/RSTP/MSTP

	Port Loop Detection
	Support

	EAPS
	Support RFC3619

	ERPS
	Support G.8032/Y.1344

	LLDP
	Support LLDP & LLDP-MED

	UDLD
	Fully compatible with CISCO's UDLD protocol

	VLLP (VRRP Layer 2 Ring Protection Protocol)
	Support, used with VRRP

	IP

	ARP
	Support static and dynamic ARP

	Static routing
	Support

	VLAN Interface
	Support 32 VLAN interfaces

	Multicast

	Static Multicast MAC Address
	Support

	IGMP SNOOPING
	Support

	MLD SNOOPING
	Support

	MVR
	Support

	GMRP
	Support

	ACL

	Standard IP-based ACL
	Support

	Extended IP based ACL
	Support

	MAC IP-based ACL
	Support

	MAC ARP-based ACL
	Support

	IPv6-based ACL
	Support

	ACL Port Filtering
	Support

	Time-based ACL
	Support

	QoS 

	Port Queue Number
	8

	Port Queue Scheduling Mode
	Support WRR, RR, WDRR, SP scheduling modes

	Port-based Classification
	Support

	802.1p-based Classification
	Support

	DSCP-based Classification
	Support

	ACL-based Classification
	Support

	QoS Policy
	Supports mapping different data streams to different queues

Supports modifying COS or DSCP values of data streams

Supports speed limiting of data streams

Supports statistics of data streams

Supports mirroring of data streams

	DHCP

	DHCPv4 Client 
	Support

	DHCPv6 Client 
	Support

	DHCP Snooping
	Support

	DHCP Relay
	Support

	DHCP Server
	Support

	DHCP option 82
	Support

	Management

	CLI  Management
	Support serial port management switch

Support TELNET management switch

Support SSH management switch

	WEB Management
	Support HTTP and HTTPS

	SNMP Management
	Support SNMP protocol v1, v2c

Support SNMP TRAP

Support a variety of standard and private MIB

	User Management
	Support multi-user management

	TACACS+
	support switch authentication via TACACS+ server remote username and password

Support password encryption in PAP and CHAP mode

Support TACACS+ server to authorize the switch’s commands

Support TACACS+ based on IPv4 and IPv6

	Log Management
	Support local log management

Support SYSLOG based on IPv4 and IPv6

	RMON
	Support RMON 1, 2, 3 and 9 groups

	Cluster Management
	Support NDP

Support NTDP

Support manual and automatic joining to cluster groups

Support unified cluster management

	Configuration file upload and download
	Support



	Upgrade software
	Support 

	Clock Management
	Support local clock and SNTP protocol

	Security

	Switch Management Security
	Supports opening and closing TELNET, WEB and SNMP services

Supports binding TELNET, WEB and SNMP services with standard IP ACL

Supports limiting the number of TELNET connections

	CPU Protection 
	The switch itself is protected against attacks by large data flows.

	AAA
	Support 802.1x protocol

Support RADIUS protocol

Support authentication, authorization and accounting through RADIUS server

Support port-based and MAC-based 802.1x

	IP MAC Binding
	Support static configuration of IP, MAC and port binding

	DHCP SNOOPING
	Support dynamic ARP binding to prevent ARP spoofing

Support dynamic IP, MAC and port binding

Support fixed port to connect to DHCP server to prevent private connection to DHCP server

	Prevent ARP Spoofing
	Supports manual configuration of ACL rules based on MAC ARP to prevent ARP spoofing

Supports DHCP SNOOPING function, during the process of obtaining IP address through DHCP, the switch dynamically binds ARP to the port to prevent ARP spoofing

	IPv6

	IPv4/IPv6 Dual Protocol Stack
	Support

	IPv6 Address
	Support manual address configuration, stateless address auto-configuration and stateful address configuration obtained through DHCPv6 client

	IPv6 Neighbor Discovery


	Support

	ICMPv6
	Support

	IPv6 Path MTU Discovery
	Support

	Debugging

	PING
	Support

	TRACEROUTE
	Support

	TELNET Client
	Support TELNET client based IPv4 and IPv6



